Problem and Motivation

Throughout the multiple research papers we have studied which cover COTS mmWave radar, FMCW SDR-based systems, 24 GHz beamforming CMOS radar, passive radar, LiDAR with PointPillars, CNN classifiers on Range-Doppler maps, micro-Doppler OMP classification, rotor-body echo separation, YOLO on RD graphs, and two broad surveys most covered how using different techniques can improve obstacle or object classification among UAV drones. Moreover, some of these research papers also show that classification accuracy reaches up to 90% in ideal situations. They proved this in various simulation experiments using different software such as MATLAB, AirSim, DEMORAD Radars, and so on. However, most of these simulations were conducted under ideal situations where different external factors do not hinder the success rate of UAVs. Our motivation is to study this very gap: how these external and environmental factors affect the mission success rate, for example, when clutter, low signals, interference, or environmental noise pushes the UAV success rate into failure.

Research Gap

The research gap this report focuses on is how different environmental factors are perceived by UAVs and how this affects their swarm formation and mission success rate. Moreover, most perception errors are widely ignored, additionally ignoring the direct relationship between swarm formation and different perception levels. This induces a factor of uncertainty in the UAVs, affecting swarm flight and increasing the risk of collision. Our main gap to study is how perception-level uncertainty affects swarm-level safety and performance.

Project Objective

The main aim and objective of this project was the study the behavior of UAVs under different external factors such that how different environmental factors, false negative, false positives, sensor dropouts and other conditions affect our UAVs swarm formations and the mission success rate as well.

Final Architecture

The Architecture of 2D Simulation written in python3 using different libraries such as numpy, pandas, matplotlib, tkinter and so on acts as the underlying code responsible for running the whole simulations and extending it to different various scenarios. The Simulation class acts as the main base claas responsible for executing the whole world, inducing motion into the UAVs driving them towards the target, checking the obstacle detection as well the avoidance logic and how adding different factors affect the success rate of the UAVs. Moving forward we improved our UAV detection logic by adding different sensors like model such as radar-like-model , vision-like-model and lidar-like-model, while the simulation is 2D based therefore completely exhibiting the use of these sensors might be not possible but the underlying logic makes them to act as the real world sensors as much as possible.

- Radar Like Sensing Model :

- The radar like model works as a simple real life like radar, it detects the surrounding obstacles within the range and if the obstacle lies in the range it records it positions and changes the direction of the UAV accordingly.

- P_D, P_FA, Clutter, Noise, Latency and Dropout

- After getting the absolute position of the obstacle, the measurements are passed through the preception class and other  functions through which we inject the errors into our measurements to exhibit the real life like UAV behaviors and record the results. These errors may included P_D(Probability of Detection), P_FA(Probability of False Alarm) , clutter(phantoms), noise(External interference), Latency(time took to send and receive the signals) and sensor dropouts which can occur randomly inside our simulation acting as causing a delay to the information received by the UAV.

- Tracking and Data Association

- The RadarTracker/RadarTrack classes in radar_track_model.py handle this. Every raw radar detection is just a noisy point in space with no identity attached  data association is the step that decides which detection belongs to which existing track, so the tracker isn't fooled into thinking a target teleported or that noise is a new object. It does this with simple nearest-neighbor gating: each existing track predicts where it should be this step (coasting forward on its last known velocity), and any detection within a set distance of that prediction gets claimed, closest pairs first, so no detection or track is used twice in one step. Once matched, a track's position and velocity are smoothed toward the new detection rather.

- Multimodal Sensing

- In this project we don’t use any form of Multimodel sensing, Instead we use different fusion modes and different results  and detection combination to make our predictions more accurate. We attach a confidence value and trust value to each detection data received from all the UAVs, and perform weighted avg, while also performing the mean of confidence and trust to determine how much we can rely on the data we received.

- Fusion Modes

- There are 6 Fusion modes we are using in our simulations.

- No Fusion

- In No Fusion Mode we don’t Fuse the results obtained from different UAVs, thus     each UAV acts as a standalone UAV.

- Naive Fusion

- In Naive Fusion we simple average the results obtained from UAVs and sharing it back making the next decision of the UAVs directions more accurate.

- Confidence Weighted Fusion

- In the Confidence weighted fusion instead of simply averaging all the values we add a confidence factor to determine how much we can trust the result.

- Trust Weighted Fusion

- It is similar to confidence wighted fusion, but the value of the weight is determined by weight = confidence * track-status-reliability * a composite. We add a reliability factor to make sure the value is reliable or not.

- Covariance Weighted Fusion

- Covariance Weighted Fusion instead of just averaging positions, each source gets weighted by how uncertain it is: a track that's noisy/unreliable (bigger "spread" of possible error) counts for less, a tight/confident one counts for more. This is the mathematically ideal way to combine sources, but only if you can assume their errors are all independent of each other.

- Covariance Intersection Fusion

- This works on the same idea as the Covariance Intersection Fusion, but it takes a bit of safer approach to prevent overfitting the data.

- Centralized and Distributed Fusion

- Centralized Fusion

- Every UAV sends its track to one central node (ground station or lead UAV), which fuses everything into a single value and broadcasts it back out. Everyone ends up with the same estimate, but nothing is usable until that round trip finishes.

- Distributed Fusion

- Each UAV keeps its own track, broadcasts a lightweight summary to the rest of the swarm, and fuses whatever peer summaries actually arrive (each broadcast can independently fail to reach a given UAV). Since delivery isn't guaranteed to be the same for everyone, different UAVs can end up with slightly different local estimates of the same object at the same step.

- communication uncertainty

- dynamic trust

- experiment design

Metrics

- statistical results

- ablation results

- stress-test results

Limitations

Reproducibility

Conclusion